Instructions for changing a scan head

- Turn off power to the scan heads (open L1-1S fuse or shutdown cabinet)

- When removing the heads from the scan frame, make sure you have the silver mounts below the scanners to prevent them from sliding off when unbolted

- Disconnect the threaded connector from the back, unbolt, then reverse to install the new head

- Power the system back on

- In the software, starting from the main menu, choose Setup Options

- Scanner Setup

- Head S/N Assignment

- The new scan head should have the serial number printed in large numbers and will likely be 6 digits starting with 1xxxxx. E.g. 111111, if you’re swapping head 10, change 88824 to 111111, then hit submit.

- Now you’ll need to close and reopen the software.

- When you swap a scan head, it’s best to calibrate right away.